STEMM Lab

Student Activity Guide

STEMM Lab is a mobile app that turns real-world physical activities into hands-on Science, Technology, Engineering, Mathematics, and Medicine (STEMM) challenges. You will use your phone's camera, sensors, GPS, and timers to collect data, compete on leaderboards, and improve your designs — just like real scientists and engineers.

## Getting Started — App Setup

When you first open the app, enter the following details for your team:

- Team Name

- First Name of each team member

- Grade or Year Level

- Team Discriminator (assigned automatically by the app)

For each activity you will:

- Follow the on-screen instructions

- Upload a video of your attempt

- Record your measurements and results in the app

- Rate the activity and write a short comment

- Compare your results on the leaderboard

# Section 1: Engineering Challenges

These activities explore forces, sound, air movement, and structural design using everyday materials and your phone sensors.

## Activity 1: Parachute Drop Challenge

Engineering + Physics

### What This Activity Is About

You will design, build, and test a parachute for a small toy to reduce its landing speed and impact force. You then compare designs through repeated testing, just like real aerospace engineers.

### Materials Needed

- Mobile phone with STEMM Lab app

- Small toy (e.g. plastic soldier)

- Table or other elevated surface (consistent drop height)

- Paper or plastic sheet (for the parachute canopy)

- String

- Scissors

- Tape

### Step-by-Step Instructions

- Drop the toy WITHOUT a parachute from your chosen height. Record this fall as your baseline.

- Build a parachute by attaching string to the corners of a sheet of paper or plastic, then tie the other ends to the toy.

- Drop the toy WITH your parachute from the same height. Record the fall using the app.

- Review the speed and landing accuracy results shown in the app.

- Redesign your parachute and test up to three different prototypes within 20 minutes.

- Upload your videos, recorded results, and team reflections to the app.

### Key Formulas

| What to Calculate | Formula | Notes |
| --- | --- | --- |
| Final velocity (speed at landing) | v = distance ÷ time | Drop height ÷ fall time; initial velocity = 0 |
| Acceleration | a = v ÷ time | Since object starts from rest |
| Net Force | F = mass × acceleration | Total force acting on the toy |
| Weight (downward force) | W = mass × 9.8 | Gravity pulling the toy down |
| Drag Force (parachute resistance) | Drag = Weight − Net Force | Force the parachute pushes up with |
| G-force (no bounce) | g-force = (v ÷ contact time) ÷ 9.8 | How hard the toy hits the ground |
| G-force (with bounce) | g-force = ((v_down + v_up) ÷ contact time) ÷ 9.8 | Add rebound speed to impact speed |

### Example Calculation

Drop height: 1.0 m | Fall time: 0.5 s | Toy mass: 0.20 kg

- Final velocity = 1.0 ÷ 0.5 = 2.0 m/s

- Acceleration = 2.0 ÷ 0.5 = 4.0 m/s²

- Net Force = 0.20 × 4.0 = 0.80 N

- Weight = 0.20 × 9.8 = 1.96 N

- Drag Force = 1.96 − 0.80 = 1.16 N

### G-Force Reference Guide

| G-Force Range | Real-World Example | Effect on the Body |
| --- | --- | --- |
| 1–5 g | Elevator, amusement ride | No injury |
| 5–10 g | Hard fall while running | Possible bruising |
| 10–30 g | Sports collision, bicycle crash | Serious injuries possible |
| 30–50 g | Severe car crash | High risk of severe injury |
| 50+ g | Very sudden stops with no cushioning | Life-threatening injuries likely |

### What Level Should I Do?

| Student Level | Focus |
| --- | --- |
| Primary School | Measure fall time. Calculate final speed only. |
| High School | Calculate velocity, acceleration, net force, drag force, and g-force. |

## Activity 2: Sound Pollution Hunter

Environmental Science

### What This Activity Is About

You will use your phone as a sound meter to measure how loud different everyday actions and locations are — then decide whether your classroom is a safe listening environment.

### Materials Needed

- Mobile phone with STEMM Lab app (uses built-in microphone)

- No other materials required

### Step-by-Step Instructions

- Choose 3 or more different sounds to test (e.g. dropping a book, talking, stamping feet, walking).

- Open the Sound Meter screen in the STEMM Lab app.

- Perform each action and hold the phone nearby. The app records the decibel (dB) level.

- Note the location and GPS tag for each reading.

- Map which areas or actions are the loudest and which are the quietest.

- Record all results in your write-up table.

### Sound Level Reference Guide

| Sound Level (dB) | Example | Risk to Hearing |
| --- | --- | --- |
| 0–30 dB | Whisper, quiet library | No risk |
| 30–60 dB | Normal conversation, classroom | Safe for long periods |
| 60–85 dB | Busy traffic, vacuum cleaner | Generally safe; long exposure causes fatigue |
| 85–90 dB | Lawn mower, loud classroom | Hearing damage possible after long exposure |
| 90–100 dB | Motorbike, power tools | Hearing damage likely after short exposure |
| 100–110 dB | Nightclub, chainsaw | Serious hearing damage in minutes |
| 110–120 dB | Siren close by | Painful; immediate damage possible |
| 120+ dB | Jet engine, explosion | Instant, permanent hearing damage |

### No Formula Required

The app measures sound automatically in decibels (dB). Your job is to compare readings and decide: should your class wear ear protection?

## Activity 3: Hand Fan Challenge

Physics – Air Movement

### What This Activity Is About

You will build different paper fans and test how their design and distance affect how much they bend a standing sheet of paper. This teaches you about air force, material stiffness, and flexible structures.

### Materials Needed

- Several sheets of paper and cardboard

- Scissors

- Sticky tape

- Ruler (to measure distance from fan to target)

- Mobile phone with STEMM Lab app (to record video and measure bend angle)

### Step-by-Step Instructions

- Stand a sheet of paper upright on the table (lean it against something or tape the base lightly).

- Make your first fan design (e.g. folded accordion style with 1 cm folds).

- Hold the fan 30 cm away and fan air at the standing paper. Observe and measure the bend angle.

- Record your result in the table.

- Repeat with the fan at 15 cm and 45 cm.

- Try a second and third fan design. Record each result.

- Replace the paper target with cardboard. Repeat and compare.

### Key Formula

To estimate the force your fan applies:

| Symbol | Meaning | Notes |
| --- | --- | --- |
| F | Force (Newtons) | Force applied by the fan air |
| k | Stiffness coefficient | Resistance of the material to bending |
| θ (theta) | Bend angle (radians) | Convert degrees: radians = degrees × π ÷ 180 |

Formula:  F ≈ k × θ

### Approximate Stiffness Values (k) for Classroom Materials

| Material | Thickness | k Value (N/rad) | Behaviour |
| --- | --- | --- | --- |
| Thin printer paper | 0.1 mm | 0.05 | Bends very easily |
| Standard card stock | 0.25 mm | 0.20 | Moderate bend |
| Thin cardboard | 0.5 mm | 0.50 | Much harder to bend |
| Corrugated cardboard | 3 mm | 2–3 | Almost no visible bend |

### Example Calculation

Thin paper (k = 0.05), observed bend angle = 30°

- Convert: θ = 30 × π ÷ 180 ≈ 0.524 radians

- Force = 0.05 × 0.524 ≈ 0.026 N

Thin cardboard (k = 0.50), same bend angle:

- Force = 0.50 × 0.524 ≈ 0.26 N

Conclusion: stiffer materials require about 10× more force for the same bend.

## Activity 4: Earthquake-Resistant Structure

Engineering + Earth Science

### What This Activity Is About

You will build a structure from paper and cardboard that can absorb vibration — simulating how earthquake engineers design buildings to protect people.

### Materials Needed

- Cardboard sheets

- Paper

- Scissors

- Sticky tape

- Plastic or paper cups (optional, for pillars)

- Mobile phone with STEMM Lab app (vibration sensor mode)

### Step-by-Step Instructions

- Fold paper or cardboard into an anti-vibration layer (your base absorber). Try different fold counts.

- Place a flat cardboard platform on top of your folded layer.

- Place the phone in the CENTRE of the platform.

- In the STEMM Lab app, activate Vibration Sensor mode. The phone screen will show how much it moves.

- Record how much the phone moves side to side (in cm).

- Modify your structure — add more pillars, increase folds, change spacing — and test again.

- Find the design that keeps the phone the most still.

### What the App Measures

| Sensor Reading | What It Tells You |
| --- | --- |
| Movement (cm) | How far the phone shifted sideways — lower is better |
| Vibration intensity | Overall shake level during the test |

### No Complex Formula Required

Compare results across designs using the movement readings from the app. The goal is to find the design pattern (folds + pillars) that reduces movement the most. The app handles the sensor maths.

# Section 2: Health and Medical Sciences

These activities explore how the human body moves, reacts, and breathes — using your phone as a scientific instrument.

## Activity 5: Human Performance Lab – Stretch Speed & Gracefulness

Medical Science + Biomechanics

### What This Activity Is About

You will measure how smoothly and quickly you can move your body during guided stretching exercises. The phone sensor detects vibration — the less vibration, the more controlled and graceful your movement.

### Materials Needed

- Mobile phone with STEMM Lab app

- Open, clear space to move safely

### Step-by-Step Instructions

- Hold the phone FIRMLY in one hand.

- Open the Vibration Sensor screen in the STEMM Lab app.

- Follow the on-screen movement guide — perform each stretch slowly and smoothly.

- The app records vibration as you move. A low reading means smooth, controlled movement.

- Repeat each movement with the vibration feedback visible so you can self-correct in real time.

- Review your speed, smoothness, and range-of-motion data after each attempt.

- Upload your results and discuss with your team which movements were hardest to control.

### Movements to Perform

| Movement | Description | Focus |
| --- | --- | --- |
| Movement 1 | Slow arm raise forward and back | Shoulder and core stability |
| Movement 2 | Slow side bend (left and right) | Trunk flexibility and balance |
| Movement 3 | Slow squat hold | Leg strength and lower body control |

### What the App Measures

| Metric | Meaning |
| --- | --- |
| Vibration (mm) | How much the phone shook — lower means smoother movement |
| Time (seconds) | How long the movement took — compare speed vs smoothness trade-off |
| Range of Motion | How far you moved in each direction |

## Activity 6: Reaction Board Challenge

Neuroscience + Mathematics

### What This Activity Is About

You will test how fast your brain can process a signal and send a command to your hands. You will compare your dominant and non-dominant hand, and track improvement across multiple attempts.

### Materials Needed

- Mobile phone with STEMM Lab app

- Clear working space

### Step-by-Step Instructions

Phase 1 – Tap Reaction (Dominant Hand)

- Open the Reaction Test screen in the app.

- Watch the screen carefully. Tap the hidden button the INSTANT it appears.

- The app records your reaction time in milliseconds.

- Each team member takes a turn. Record all results.

Phase 2 – Non-Dominant Hand

- Repeat all of Phase 1, but use your non-dominant hand.

- Compare your times between hands.

Phase 3 – Tracing Challenge

- A moving shape appears on the screen.

- Trace it as accurately and quickly as you can.

- The app measures your accuracy (how close you stayed to the line) and delay (how far behind you fell).

### Key Concepts

| Term | What It Means |
| --- | --- |
| Reaction time | Time from seeing the signal to tapping — brain + nerve + muscle speed combined |
| Dominant hand | Your stronger, more practiced hand (usually faster) |
| Hand dominance | The brain's preference to use one side more — affects speed and accuracy |
| Tracing accuracy | How precisely your hand follows the moving target — tests coordination |

### How to Calculate Your Average Reaction Time

Add all your attempts together, then divide by the number of attempts.

| Formula | Example (3 attempts: 0.32 s, 0.28 s, 0.30 s) |
| --- | --- |
| Average = total ÷ number of attempts | (0.32 + 0.28 + 0.30) ÷ 3 = 0.30 seconds |

## Activity 7: Breathing Pace Trainer

Medical Science

### What This Activity Is About

You will measure your breathing rate at rest and after exercise. The phone placed on your chest detects each breath as a small movement. You will then see how exercise changes how fast and deeply you breathe.

### Materials Needed

- Mobile phone with STEMM Lab app

- Flat surface or mat to lie down on

### Step-by-Step Instructions

- Lie down or sit still on a flat surface.

- Place the phone GENTLY on your chest (face up, flat).

- Open the Breathing Sensor screen in the STEMM Lab app.

- Breathe normally for 1 minute. The app counts your breaths per minute (BPM). Record this as your RESTING rate.

- Put the phone aside and perform Exercise 1: Jog on the spot for 1 minute.

- Immediately place the phone back on your chest and record your breathing rate again.

- Rest briefly, then perform Exercise 2: 100 star jumps.

- Record your breathing rate after this exercise too.

- Each team member rotates through steps 1–8.

### Normal Breathing Rate Reference

| State | Typical Breaths Per Minute | Notes |
| --- | --- | --- |
| Resting (adult) | 12–20 breaths/min | Lower with fitness training |
| Resting (child) | 18–25 breaths/min | Naturally faster than adults |
| Light exercise | 20–30 breaths/min | Body starts needing more oxygen |
| Intense exercise | 40–60 breaths/min | Maximum demand on lungs and heart |

### Formula: Breaths Per Minute

If the app records a 30-second window:

| Formula | Example |
| --- | --- |
| Breaths per minute = breath count × 2 | 14 breaths in 30 s → 14 × 2 = 28 breaths/min |

### Why Does Breathing Rate Increase?

During exercise your muscles need more oxygen to produce energy. Your brain signals your lungs to breathe faster and deeper to supply that extra oxygen — and to remove the extra carbon dioxide your muscles produce. The STEMM Lab app lets you see this process happening in real time on your chest.

# Quick Reference: All Activities at a Glance

| Activity | Topic | Key Material(s) | App Feature Used | Main Formula / Measurement |
| --- | --- | --- | --- | --- |
| 1 – Parachute Drop | Engineering / Physics | Paper, string, toy | Camera / Timer | F = ma; Drag = Weight − Net Force |
| 2 – Sound Pollution | Environmental Science | Phone only | Microphone / dB meter | Read dB directly from app |
| 3 – Hand Fan | Physics (Air / Force) | Paper, cardboard, tape | Camera / angle measure | F ≈ k × θ |
| 4 – Earthquake Structure | Engineering / Earth Science | Cardboard, cups, tape | Vibration sensor | Compare movement (cm) across designs |
| 5 – Stretch & Gracefulness | Biomechanics / Medical | Phone, open space | Vibration sensor | Compare vibration (mm) and time (s) |
| 6 – Reaction Board | Neuroscience / Maths | Phone only | Reaction timer / tracing | Average = total ÷ attempts |
| 7 – Breathing Trainer | Medical Science | Phone, flat surface | Breathing sensor | BPM = count × 2 (if 30 s window) |

## General Tips for Every Activity

- Always complete a BASELINE measurement first (before building or exercising) so you have something to compare against.

- Repeat each test at least 3 times — results can vary, and averaging gives a more reliable answer.

- Use slow-motion video in the app when you need to measure very short time intervals (e.g. bounce contact time).

- Take photos or short notes of each design change so you can explain WHY one design worked better than another.

- GPS tagging is automatic — the app logs your location for each recording.

- Upload your video, results, and reflection after EVERY attempt — not just the best one.

Good luck, STEMM scientists! 🔬